| Features - AEC-Q100 Grade 2 Qualified - 1KHz, 12-bit PWM Dimming resolution - Temperature compensation for red channel - 8-bit brightness resolution for red, green, and blue LED - Maximum of 4079 devices in one LED chain - Bidirectional, half-duplex, 2Mbps, CRC protected serial communication - Support for 16 multicast address groups - On-die oscillator - Built-in diagnostic functions Applications - Ambient lighting - Converting of single ended to differential bus signals |  | General Description This device is designed to intelligently respond to the sophisticated ambient lighting scenarios in evolving automotive interiors. It incorporates a fully calibrated package containing three-color RGB LED chips and drive IC, ensuring precise calibration before shipment. Through a single microcontroller, up to 4079 devices can be daisy-chained and controlled. Moreover, the microcontroller can detect the temperature and fault status of each device. The addition of CAN transceivers between daisy-chained devices ensures seamless long-distance data communication. Ordering Information Product ID; Package; Body Size. ID804; XXX; 3x3x0.9mm. |
| --- | --- | --- |

ID804 ( RGB LED Driver IC for Smart Ambient Light )

| Features - AEC-Q100 Grade 2 Qualified - 1KHz, 12-bit PWM Dimming resolution - Temperature compensation for red channel - 8-bit brightness resolution for red, green, and blue LED - Maximum of 4079 devices in one LED chain - Bidirectional, half-duplex, 2Mbps, CRC protected serial communication - Support for 16 multicast address groups - On-die oscillator - Built-in diagnostic functions Applications - Ambient lighting - Converting of single ended to differential bus signals |  | General Description This device is designed to intelligently respond to the sophisticated ambient lighting scenarios in evolving automotive interiors. It incorporates a fully calibrated package containing three-color RGB LED chips and drive IC, ensuring precise calibration before shipment. Through a single microcontroller, up to 4079 devices can be daisy-chained and controlled. Moreover, the microcontroller can detect the temperature and fault status of each device. The addition of CAN transceivers between daisy-chained devices ensures seamless long-distance data communication. Ordering Information Product ID; Package; Body Size. ID804; XXX; 3x3x0.9mm. |
| --- | --- | --- |

Revision History

| Date (YYYY-MM-DD) | Ver. | Editor | Contents |
| --- | --- | --- | --- |
| 2023/09/19 | 0.1 | BC. CHO | Released with: - Preliminary datasheet |
| 2023/10/13 | 0.2 | Keon Lee | Updated for: - Table of Electrical parameters |
| 2023/12/21 | 0.3 | YS. KIM BC. CHO | Updated for: - Description of the IC Functional & Register map |
| 2024/04/23 | 0.44 | BC. CHO | Updated for: - Phase Shift |
| 2024/05/03 | 0.45 | BC.CHO | Updated for: - Red LED Temperature Compensation method |
| 2024/05/17 | 0.5 | Keon Lee | Updated for: - Description of the Block Diagram & Pin Configuration |

Contents

1.	Block Diagram	4

2.	Pin Configuration	5

2.1.	Pin Map	5

2.2.	Pin Description	5

3.	General Product Characteristics	6

3.1.	Absolute Maximum Rating	6

3.2.	Recommended Operating Condition	6

3.3.	Electrical Characteristic	7

3.4.	Thermal Considerations	11

4.	Functional Description	12

4.1.	Feature Description	12

4.1.1.	Device States	12

4.1.2.	Device Enable	13

4.2.	Communication	14

4.2.1.	Single Ended, LVDS Link and Data Structure	14

4.2.2.	Communication mode selection	14

4.2.3.	MCU-mode communication physical layer and bit coding	15

4.2.4.	LVDS-mode communication physical layer and bit coding	15

4.2.5.	CAN-mode communication physical layer and bit coding	15

4.2.6.	Message Frame Format	16

4.2.7.	Message Handling	17

4.3.	Color Control and Temperature Compensation	20

4.3.1.	PWM Generation	20

4.3.2.	Brightness Control Scaler	21

4.3.3.	Temperature Compensation (TBD)	21

4.4.	Diagnostics	22

4.4.1.	Fault Reaction	23

4.5.	ID804 Commands	24

5.	Typical Application Circuit	34

5.1.	Layout Guidelines	35

6.	Package Information	35

Block Diagram

Figure 1. Functional Block diagram

The device implements a communication for the reception of control commands and for providing device status and configuration data. Low side, configurable constant current sinks are provided for controlling 3 LEDs (i.g. RGB). The main unit computes the PWM duty cycles from the incoming commands and applies the corresponding control values to the three PWM units. The main unit is also in charge of a periodic temperature measurement and an appropriate duty cycle adjustment for the red PWM channel. The actual device temperature is obtained via an integrated analog-digital converter (ADC). Besides the temperature the ADC can also measure various other analog values. These measurements are always triggered by a command from the host. The result of the corresponding A/D conversion is also retrieved by a host command. As each device is individually calibrated to compensate for production variations, the corresponding parameters can be stored in an on-die non-volatile memory. This one-time-programmable memory (OTP) is read at hardware reset and the parameters are copied from the OTP to directly accessible registers.

Pin Configuration

Pin Map

Figure 2. Top-View Pin Map and Electrical Internal Circuit

Pin Description

| Pin No | Pin Name | I/O | Description |
| --- | --- | --- | --- |
| 1 | VLED | Power | RGB LEDs Anode side |
| 2 | VDD | Power | IC supply voltage |
| 3 | SIO1_P | I/O | Serial communication interface mater side, positive polarity |
| 4 | SIO1_N | I/O | Serial communication interface mater side, negative polarity |
| 5 | PGND | IN | Power Ground |
| 6 | TEST | IN | TEST |
| 7 | TM/EXT | I/O | TM or For measuring of External analog voltage |
| 8 | PGND | IN | Power Ground |
| 9 | SIO2_N | I/O | Serial communication interface slave side, negative polarity |
| 10 | SIO2_P | I/O | Serial communication interface slave side, positive polarity |
| 11 | VDD | Power | IC supply voltage |

General Product Characteristics

Absolute Maximum Rating

The absolute maximum ratings define values beyond which damage to the device may occur. Exposure to absolute maximum rating conditions for extended periods may affect device reliability. The functional operation of the device at these or any other conditions beyond the recommended operating ratings is not guaranteed.

Over operating free-air temperature range (unless otherwise noted)(1)

| Parameter | Pin | MIN | MAX | UNIT |
| --- | --- | --- | --- | --- |
| Input voltage range (with respect to GND unless specified) | VDD | -0.3 | 7.0 | V |
|  | VLED | -0.3 | 7.0 | V |
|  | VEXT | -0.3 | 7.0 | V |
| Input/Output voltage range (with respect to GND unless specified) | SIO1_P/N | -0.3 | VDD+0.3 | V |
|  | SIO2_P/N | -0.3 | VDD+0.3 | V |
| Junction to ambient thermal resistance | θja | tbd |  | ℃/W |
| Junction to case thermal resistance | θjc | tbd |  | ℃/W |
| Storage temperature | Tstg | -40 | 125 | °C |
| junction temperature | TJ | -40 | 125 | °C |
| ESD | HBM(2) | ±2000 |  | V |
|  | CDM(3) | ±500 |  | V |
|  | CDM (Corner pins) | ±750 |  | V |

- Stresses beyond those listed under Absolute Maximum Ratings may cause permanent damage to the device. These are stress ratings only, and do not imply functional operation of the device at these or any other conditions beyond those indicated under Recommended Operating Conditions. Exposure to absolute- maximum-rated conditions for extended periods may affect device reliability.

- AEC Q100-002 indicates that HBM stressing shall be in accordance with the ANSI/ESDA/JEDEC JS-001 specification.

- AEC Q100-011

Recommended Operating Condition

| Parameter | Description | Min. | Typ | Max. | Unit |
| --- | --- | --- | --- | --- | --- |
| VVDD, VLED | DC supply voltage IC and LED | 4.3 | 5 | 5.5 | V |
| TINIT | Device ready for commands, Initialization time after VDD > VUVLO level |  |  | 1 | ms |
| fCOMM | Data rate for communication |  | 2.0 |  | Mbps |
| VTM | Programming voltage** | - | 0 | 30 | mV |
| TA | Ambient temperature | -40 |  | 105 | ℃ |

Electrical Characteristic

The ◎ denotes the specification which apply over the specified operating junction temperature range, otherwise specifications are for TA = 25°C, VDD=5V, CVDD=1μF, unless otherwise noted.

| Symbol | Parameter | Test Conditions | 3T | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VDD BIAS SUPPLY |  |  |  |  |  |  |  |
| VPOR | Power-On-Reset (POR) | VDD voltage capable of maintaining logic data | ◎ |  |  | 3.8 | V |
| Vuvth_hi | Undervoltage Lockout threshold | VDD Rising, LED driver is enable | ◎ | 4.1 | TBD | 4.3 | V |
| Vuvth_lo |  | VDD Falling, LED driver is disable | ◎ | 3.9 | TBD | 4.1 | V |
| Vuvdet_hyst | UVLO Detection hysteresis threshold | Guaranteed by design |  |  | 180 |  | mV |
| DC Supply Currents |  |  |  |  |  |  |  |
| IQ_dpslp | VDD deep sleep mode current | Deep Sleep mode, TA = 25°C |  |  | 250 |  | uA |
| IQ_sleep | VDD sleep mode current | Sleep mode, LED driver off | ◎ |  | 0.8 |  | mA |
| IQ_activ | VDD active mode current | Active mode, LED driver on | ◎ | 1.0 + LEDs current*0.11 |  |  | mA |
| RGB LED Driving current characteristic |  |  |  |  |  |  |  |
| IO_LED_acc | Constant output current accuracy | All LED is on, VOUTn=1V, IDAC=max | ◎ | -5% | 20 | +5% | mA |
| IO_LED_drift | Constant output current aging drift |  |  | -1% |  | +1% | % |
| VO_LED_hdrm | Constant output current headroom voltage | IDAC=max | ◎ |  |  | 0.7 | V |
| IO_LED_ldrg | Load regulation of constant-current output | All LED is on, VOUTn=1V to 3V, IDAC=max | ◎ | -1% |  | +1% | %/V |
| IO_OFF_lkg | Output leakage current | All LED is off, VOUTn=5V | ◎ |  | 0.1 | 1 | uA |
| RGB LED Driving Current Temperature Drift ( *ΔIi = Abs((I25°C@ 2V – ITemperature) / I25°C@ 2V)) |  |  |  |  |  |  |  |
| ΔIRED* | RED Driving current temperature drift | TA=-40℃, Max current |  |  |  | 5 | % |
|  |  | TA =105℃, Max current |  |  |  | 5 |  |
| ΔIGREEN* | GREEN Driving current temperature drift | TA =-40℃, Max current |  |  |  | 5 | % |
|  |  | TA =105℃, Max current |  |  |  | 5 |  |
| ΔIBLUE* | BLUE Driving current temperature drift | TA =-40℃, Max current |  |  |  | 5 | % |
|  |  | TA =105℃, Max current |  |  |  | 5 |  |
| Dimming parameter relationships between RGB parameters and PWM frequencies |  |  |  |  |  |  |  |
| TSR_RS_LED | LED current Rising time | IOUTn (10% to 90%), Guaranteed by design |  |  | 0.3 | 1 | us |
| TSR_RS_LED | LED current Falling time | IOUTn (90% to 10%), Guaranteed by design |  |  | 0.3 | 1 | us |
| TDLY_ON_LED | LED On command Propagation delay time | From end of command to 10% IOUTn, Guaranteed by design |  |  | 30 | 50 | ns |
| TDLY_OFF_LED | LED Off command Propagation delay time | From end of command to 90% IOUTn, Guaranteed by design |  |  | 30 | 50 | ns |
| Dimming Phase shift for RGB LED driving |  |  |  |  |  |  |  |
| PSHIFT_Green | Green Phase shift refer to PWM dimming freq | Guaranteed by design |  |  | 0 |  | degree |
| PSHIFT_Red | Red Phase shift refer to PWM dimming freq | Guaranteed by design |  |  | 90 |  | degree |
| PSHIFT_Blue | Blue Phase shift refer to PWM dimming freq | Guaranteed by design |  |  | 270 |  | degree |
| LED Fault diagnostic |  |  |  |  |  |  |  |
| VTH_OPEN | LED Open diagnosis threshold voltage |  | ◎ |  | 250 |  | mV |
| VTH_Hys_OPEN | LED Open hysteresis threshold voltage |  |  |  | 30 |  | mV |
| VTH_SHRT | LED Short diagnosis threshold voltage | SH_LVL[1:0] = 2b’01 | ◎ |  | 4 |  | V |
| Tdiag_filter | LED fault diagnostic deglitch filter count | Design info. Counter clock is based on “F_PWM_DIV[3:0]” configuration |  |  | 16 |  | cnt |
| Main Oscillator |  |  |  |  |  |  |  |
| Fosc_main | Internal Main Oscillator frequency |  | ◎ | 14.4 (-10%) | 16 | 17.6 (+10%) | MHz |
| Serial Interface |  |  |  |  |  |  |  |
| fcomm_data_input | Allowed data rate for communication input |  |  | 1.8 (-10%) | 2 | 2.2 (+10%) | MHz |
| fME_data_sing | Signal data rate for single ended Manchester code | Manchester coded signal = Real data rate x 2 |  |  |  | 4.4 | Mbps |
| Dcomm_data_input | Allowed duty cycle variance for communication input |  |  | 45 |  | 55 | % |
| tmsg_interval | Time between two messages | Send from MCU |  |  | 6 (12clk) |  | us |
| Single-ended Physical layer parameters |  |  |  |  |  |  |  |
| VIH,se | input high level voltage |  | ◎ | 2.0 | - |  | V |
| VIL,se | input low level voltage |  | ◎ |  | - | 0.8 | V |
| ROUT_PU_SE | Output driver pull-up resistance | IDRV_OUT= -10mA | ◎ |  | 35 | tbd | Ω |
| ROUT_PD_SE | Output driver pull-down resistance | IDRV_OUT= 10mA | ◎ |  | 35 | tbd | Ω |
| TDLY_RXTX_SING | Propagation delay from RX to TX on single-ended | Low to High | ◎ |  |  | 50 | ns |
|  |  | High to Low | ◎ |  |  | 50 | ns |
| LVDS Physical layer parameters |  |  |  |  |  |  |  |
| VLVDS_comm | Common mode voltage with active communication |  | ◎ |  | 1.2 |  | V |
| VLVDS_off | Common mode voltage with no active communication | Design info. |  |  | 0 |  | V |
| VLVDS_diff | LVDS differential voltage | Design info. |  |  | 300 |  | mV |
| RLVDS_term | LVDS termination resistance | Included IC | ◎ | -10% | 200 | +10% | Ohm |
| ILVDS_TX | LVDS TX Current |  | ◎ | -10% | 1.5 | +10% | mA |
| RLVDS_pd | LVDS Pull down resistance | Included IC | ◎ |  | 200K |  | Ohm |
| TDLY_RXTX_LVDS | Propagation delay from RX to TX on LVDS mode | Low to High | ◎ |  |  | 50 | ns |
|  |  | High to Low | ◎ |  |  | 50 | ns |
| Analog Digital Converter (TBD) |  |  |  |  |  |  |  |
| ADCBIT | Bit Resolution |  |  |  | 10 |  | bit |
| Fsmp_rate | Sampling Rate |  |  |  | 250K |  | SPS |
| SNR | Signal to Noise Ratio |  |  | 52 | 55 |  | dBc |
| SNDR | Signal to Noise and Distortion Ratio |  |  | 50 | 54 |  | dBc |
| SFDR | Spurious Free Dynamic Range |  |  | 60 | 62 |  | dBc |
| INL | Integral Nonlinearity |  |  | -2 |  | +2 | LSB |
| DNL | Differential Nonlinearity |  |  | -1 |  | +1 | LSB |
| ERRoffset | Offset error |  |  | -4 |  | +4 | LSB |
| Vext_input | Input Range for External | EXT pin |  | 0.50 |  | 4.50 | V |
| Vtemp_input | Input range for Temp Sen. |  |  | 0.75 |  | 1.65 | V |

Thermal Considerations

Need to suggest the maximum amount of power that the IC can consume under AEC-Q100 grade 2 conditions(TA 105°C at still air). Thermal resistance characteristics would be variable depending on the IC, package, and PCB layer characteristics.

“Example of allowable IC power dissipation characteristics by ambient temperature”

Functional Description

Feature Description

Device States

Figure 3. IC State

POR : The ID804 is in an off state.

UNINIT : The ID804 will reach this state automatically after power-up. PWM values are 0 and LED drivers disabled.

In this state, the ID804 only responds to a limited set of commands:

- INITBIDIR

- RESET (broadcast)

- CLRERROR (broadcast)

- SET_SETUP1 (broadcast)

- SET_SETUP2 (broadcast)

- SET_TEMPTH (broadcast)

- SET_TEMPHYS (broadcast)

- SET_RGB (broadcast)

Other telegrams are ignored. Broadcast telegrams are fast forwarded and addressed telegrams are forwarded after the telegram has been fully received.

SLEEP : The ID804 enters SLEEP state after receiving an INITBIDIR command, after a failure event (if selected) or via GOSLEEP command.

In this state, the LED drivers are disabled but the last PWM parameters are stored, i.e., not reset to 0. An update of the PWM parameters is possible.

ACTIVE : The ID804 enters this mode after receiving the GOACTIVE command if no fault conditions are present.

In this state the LED drivers are enabled with the respective current setting, PWM frequency and duty cycle for each channel. An update of the PWM parameters is possible.

After entering the ACTIVE state, the open/short detection is executed once for every channel with a PWM value exceeding the minimum value. See Chapter 4.4.

DEEPSLEEP : The ID804 is in a power save mode with reduced current consumption.

Device Enable

When the supply voltage is connected and VDD increases above the VPOR level, or the RESET command is received, the ID804 undergoes the following startup sequence (Figure 4Figure 4. Startup behavior of the ID804. The example shows the MCU (left) and EOL (right) modes. After a RESET command, the device starts in the PoR state.)

- All registers are initialized to their default state (PoR in Figure 4).

- Trim settings are loaded and checked for integrity. If a mismatch is detected, the OTPCRC flag in the STATUS2 register is set.

- The voltage levels at each SIOx pin are checked to detect the communication modes (shaded signals in Figure 4).

- The ID804 enters the UNINIT mode and is now ready to receive the first command (Uninitialized in Figure 4).

Figure 4. Startup behavior of the ID804. The example shows the MCU (left) and EOL (right) modes. After a RESET command, the device starts in the PoR state.

The whole sequence takes approximately 1 ms.

Communication

Single Ended, LVDS Link and Data Structure

Overview

- Bi-directional

- LVDS - differential Manchester coded

- Single ended – Manchester coded & Manchester decoded (NRZ, clock & data) Push-pull interface, clock is invertible

- Communication direction and SIOx connection is flexible

- Communication speed is typ. 2Mbps Manchester coded

Communication modes

Each one of the 2 available SIOx interfaces do support 4 different modes.

Each SIOx only can operate in semi-duplex mode meaning it either does receive or transmit a telegram at the same time. However, it is possible that a device does receive a telegram at one SIOx and start transmitting at the other SIOx interface.

The 4 communication modes are:

- “LVDS-mode”:

The SIOx communication is using LVDS signaling for transmitting and receiving a communication telegram. The bit stream is using Manchester encoding to transfer DATA & CLK on the same line.

- “MCU-mode”:

In this mode a SIOx is using single-ended mode for the signaling. Received telegrams are Manchester encoded on SIOx_P port. For transmitting, the device is decoding the Manchester signal. SIOx_P will deliver the DATA in NRZ format and SIOx_N delivers the corresponding clock.

- “EOL(End Of Line)-mode”:

SIOx port does consider itself to be at the end of the daisy chain.

- “CAN-mode”:

In this mode, it is possible to communicate via a CAN FD transceiver. SIOx_P is always in output state and transmits telegram as Manchester encoded signals (single-ended), SIOx_N is always in input state and receives telegrams as Manchester encoded signal (single-ended).

Communication mode selection

Communication mode auto detection takes place during initialization phase via voltage level check on SIOx_P & SIOx_N and this mode cannot be changed during the operation cycle. After a HW Reset (POR) or RESET command received the Initialization with mode selection take place again.

Table 1. Communication mode selection

| Mode | Pin Configuration |  | I/O |  | Signal Type | Data Encoding |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SIOx_P | SIOx_N | SIOx_P | SIOx_N |  | TX | RX |
| MCU | Pull-Up | Pull-Down | Bi-dir | Output or Inactive | Single-ended | NRZ (CLK&Data) | Manchester |
| LVDS | Pull-Down | Pull-Down | Bi-dir | Bi-dir | LVDS | Manchester | Manchester |
| CAN | Pull-Up | Pull-Up | Output | Input | Single-ended | Manchester | Manchester |
| EOL | Pull-Down | Pull-Up | - | - | - | - | - |

MCU-mode communication physical layer and bit coding

Single ended communication in the master to slave direction is performed with so-called Manchester encoded. In that case a bit stream is sent via SIOx_P only; SIOx_N of the device is ignored. Slave to master communication is performed with NRZ coding. In that case SIOx_N provides a clock signal. The clock signal is inverted if it’s selected via setup register. Communication Data rate effective is typ. 2Mbps.

Example of Communication Waveforms

Figure 5. Example of MCU to ID804 communication

The receiving clock could be inverted via setup register

Figure 6. Example of ID804 to MCU communication

LVDS-mode communication physical layer and bit coding

For differential communication the termination resistance RLVDS_term is 200 Ohm, differential voltage VLVDS_diff is typ. 300 mV, data rate is typ. 2 Mbps. Common mode voltage VLVDS_comm is typ.1.2 V, it means the max voltage on the communication lines during communication is typ. 1.35 V and the min. voltage is typ. 1.05 V

Example of Communication Waveforms

Figure 7. Example of ID804 to ID804 communication

CAN-mode communication physical layer and bit coding

If a transceiver is detected on SIOx the output and input message is via single ended Manchester encoded communication with typ. 2 Mbps data transfer rate. For the transceiver the data transfer rate depends on the data signal due to the Manchester encoded data. Means up to the double transfer rate is needed (4 Mbps). Requires a 5 Mbps transceiver

Example of Communication Waveforms

Figure 8. Example of CAN Transceiver to ID804 communication

Figure 9. Example of ID804 to CAN Transceiver communication

Message Frame Format

ID804 has 3 types of frame format according to DATA bit. The format is the same for upstream and downstream communication in case of data bit is 12 or 24. 0bit data frame format Figure 12. 0bit Data Frame FormatFigure 12 can use only downstream.

Figure 10. 24bit Data Frame Format

Figure 11. 12bit Data Frame Format

Figure 12. 0bit Data Frame Format

The bit order is MSB first.

- PREAMBLE (4 bit) : Fixed to 4’b1010. 
Avoid messages with incorrect preamble as this could lead to forwarding of garbage telegrams along the chain and to raising the STATUS2.COM_FLT.

- DEV_ADDR (12 bit) : Allowed values for downstream messages are 0x000 (broadcast) and 0x001 to 0xFEE (unicast nodes) and 0xFEF to 0xFFE (multicast). Upstream messages always contain the device address of the sender in the DEV_ADDR field. Not all commands are allowed in a broadcast message. See Chapter 4.5

- COMMAND (8 bit) : See Chapter 4.5

- DATA (24bit or 12bit or 0bit) : DATA field has bits according to COMMAND. See Chapter 4.5

- DUMMY (4bit) : In case of 12bit Data Frame Format, 4bit DUMMY data (4'b1010) inserted  to match the data size to BYTE.

- CRC (8 bit) : See Chapter 4.2.7.8

Message Handling

Broadcast

An address of 0x000 initiates a broadcast, i.e., the same message is sent and received by all nodes on the chain. Whether a command can be broadcast is detailed in Chapter 4.5.

Broadcast Read or SR command is first received by all devices via the frame in downstream direction. The last node in the chain then immediately transmits its response frame upstream. The response frame’s data field depends on the actual command. The response frame’s address field is set according to the own device’s address. All the nodes upstream forward all received response frames until a frame with the address of their adjacent node is received. Then the respective node transmits its own response frame. This procedure lasts until the chain’s first node has transmitted its response frame.

Figure 13. Example of broadcast write

Figure 14. Example of broadcast read(or SR)

Multicast

An address of 0xFEF to 0xFFE initiates a multicast, i.e., the same message is sent and received by same defined group nodes on the chain. Multicast group can be defined by MCAST command. Whether a command can be multicast is detailed in Chapter 4.5.

Forwarding and Fast Forwarding

Message forwarding refers to the direct forwarding of a message without modification.

Messages are forwarded to the next device if the address in the message frame is the broadcast address, or multicast address, or it is different from the node’s address.

If a message is to be forwarded, forwarding starts exactly one cycle after the preamble and address field are fully received, i.e., after 17 bits. This is referred to as fast-forwarding.

When a node is uninitialized, broadcast messages are fast-forwarded and multicast(0xFEF to 0xFFE) & unicast(0x001 to 0xFEE) addressed messages are forwarded only after the full telegram has been received.

Responses and SR (Status Requests)

Some commands trigger a response from the addressed node. In this case, the address field contains the address of the sending node to indicate the origin of the message. The response is typically sent out approximately 3 bit-periods after the end of the command telegram has been received by the node.

Some commands do not send a response but allow a SR(Status Request). The SR(Status Request) is done by specific command. See Chapter 4.5. In case of broadcast response or SR see Chapter 4.2.7.1.

If a valid SR(Status Request) command has been received, the node responds by sending the TEMP & STATUS2 registers to the MCU after the original command has been executed. Status bits are cleared after responding to the SR(Status Request). In the case of the CLRERROR command, the status bits are cleared by the command before the status request is sent.

Commands supporting a status request are marked in the command description. See Chapter 4.5.

Note that the ID804 does not allow simultaneous upstream and downstream telegrams on the same node, as this could lead to dropped or corrupted messages.

Waiting Time between Telegrams

ID804 is not accepting a new telegram while a telegram is being received or transmitted on the other port. Likewise, simultaneous upstream and downstream messages are not supported.

Therefore, a waiting time has to be added by the MCU between sending two telegrams in order to ensure that all receiving and transmitting activities have been concluded.

The minimum waiting time between two downstream telegrams should be 8.5 μs (edge to edge) and applies to write-only or without response request commands. If an answer has been requested, the next telegram should only be sent after the response has been received by the MCU.

Communication Errors

The ID804 requires certain communication errors to be detected by each unit in a daisy chain. The below Table 2 summarizes all relevant error conditions.

Table 2. Communication Errors

| Preamble | Address | Command | CRC*) | COM_FLT flag |
| --- | --- | --- | --- | --- |
| Incorrect | - | - | - | Set |
| Correct | Unicast | Incorrect | - | Set |
|  |  | Correct | Incorrect | Set |
|  | Multicast | Incorrect | - | Set |
|  |  | Correct | Incorrect | Set |
|  | Broadcast | Incorrect | - | Set |
|  |  | Correct | Incorrect | Set |

*) applies if CRC check is enabled

The ID804 performs some error detection even for messages that are to be forwarded(CRC). If an error is detected in such a case, the COM_FLT flag is set and the message is still forwarded, i.e., communication is not stopped.

Errors due to incorrect command is only detected if the unit is addressed by unicast or multicast or broadcast.

Communication Timeout (TBD)

The READ commands or SR WRITE commands trigger a response from the addressed or all nodes. Each node needs to await all responses originating from the nodes downstream. Thereafter either the node’s own response is transmitted or new commands are accepted. Only the last node in the LED chain may immediately transmit its response. In case there is an error with the chain, not all expected responses may arrive. Thus, each of the commands expecting a response waits for a certain time only and then returns to its previous state without having transmitted the node’s response data. The lengths of the timeouts depend on the respective command. They are calculated to account for the worst-case oscillator frequency tolerance. I.e. the waiting node has a high-speed clock and all the nodes waited for have a low speed clock. The hardware implementation uses an internal clock for the timeout counter. With the nominal clock frequency of 16MHz the counter’s resolution results to 1.024ms.

Table 3. Timeout Specification

| Command | Counter Value | Min. timeout | Typ. timeout | Max. timeout | Units |
| --- | --- | --- | --- | --- | --- |
| SR WRITE (Downstream DATA field : 0bit) | TBD | TBD | TBD | TBD | ms |
| SR WRITE (Downstream DATA field : 12bits) | TBD | TBD | TBD | TBD | ms |
| SR WRITE (Downstream DATA field : 24bits) | TBD | TBD | TBD | TBD | ms |
| READ (Upstream DATA field : 12bits) | TBD | TBD | TBD | TBD | ms |
| READ (Upstream DATA field : 24bits) | TBD | TBD | TBD | TBD | ms |

Cyclic Redundancy Check

The CRC field contains the CRC8 check word calculated over the full message excluding the CRC field.

CRC Implementation: x8 + x5 + x3 + x2 + x + 1.

Polynomial: 0x2F (normal notation)

Initial value: 0x00

XOR value: 0x00

Direction: MSB first

Note that this implementation uses the same polynomial but is not compatible with the AUTOSAR implementation because of the different initial and XOR values of 0xFF.

Note: The calculation direction for the CRC field in the non-volatile memory is LSB first.

Color Control and Temperature Compensation

PWM Generation

The ID804 incorporates three independent PWM channels, one for each LED.

The resolution is 12bit. The supported duty cycles are 0/4096 to 4095/4096. The PWM output frequency is set by SETUP1.F_PWM_DIV register setting. The frequency is decided by (Fosc_main(Typ. 16MHz)) / (F_PWM_DIV + 1). The PWM frequency is reduced to the quarter of set frequency when RGB data is less than decimal 8.

The output frequency is determined independently for each of the PWM channels.

PWM Update

When a new PWM duty cycle has to be applied, this is always done at the end of a PWM cycle. I.e. the PWM always completes an output cycle using the previously active duty cycle and starts the next output cycle using the updated duty cycle.

Phase Shift

In order to spread the current consumption of the LEDs over time, a phase shift enabled between the three PWM channels. It retains even if the output frequency of the channels is different. If a channel is operating at a lower frequency, it may be considered to leave out one or three full PWM cycles. When leaving DEEPSLEEP state, the channels are restarted appropriately to again obtain the correct phase shift.

The fixed phase shift is defined in the following Table 4. Please note the absolute phase shift times are nominal values. I.e. they are subject to vary with the internal oscillator’s frequency.

Table 4. PWM Phase Shift

| PWM Channel | Related Phase Shift | Unit |
| --- | --- | --- |
| Green | 0 | degree |
| Red | 90 | degree |
| Blue | 270 | degree |

Brightness Control Scaler

In operation, the color brightness of each LED is controlled via an 8-bit RGB register. This provides a color space of 3 x 8 bit = 24 bit. The 8-bit RGB value is scaled up to the 12-bit value using by Brightness calibration value(PWMmax value). The PWMmax values is stored OTP.

The scaling equations of the red and green and blue channel are given in Table 5

Table 5. Scaling equation

| Color | Scaling equation |
| --- | --- |
| Red | PWMRED = (PWMmax X R data + 128) >> 8 (When SETUP1.TC_E = 0, another case see Table 6) |
| Green | PWMGREEN = (PWMmax X G data + 128) >> 8 |
| Blue | PWMBLUE = (PWMmax X B data + 128) >> 8 |

Temperature Compensation (TBD)

For temperature detection, the ID804 provides an analog temperature sensor. This can be measured by ADC and value can be read from TEMP.TEMPERATURE register through READ_TEMP command.

| Parameter | Description | Equation | Unit |
| --- | --- | --- | --- |
| Tj,det | Detected junction temperature | Tj,det = (Tempoffset + XXX – XXX ) / XXX | ℃ |
| TEMPERATURE[9:0] | Detected Temperature value of ADC |  | digit |

Red LED Temperature Compensation

The luminous intensity of typical red LEDs shows a significant temperature-dependency that is almost linear. The ID804 provides a feature to compensate this effect. For that purpose, the on-chip temperature sensor measures the chip temperature. Measured temperature value is used as a point to a lookup table. The lookup table is configured by MCU. In below figure, TC_BASE and TC_OFFSET values are stored in OTP. TEMP_LUT_TCx register value is written by MCU through SET_TEMP_LUT_TCx. The lookup table coefficient(TC) is used to RED color PWM scaling like below Table 6 equation. For example, if the measured chip temperature value is in segment #5 in below figure, the TC value in Table 6 is TEMP_LUT_TC5. This temperature compensation function can be enabled or disabled by SETUP1.TC_E register setting.

Table 6. Scaling equation with RED Temperature Compensation

| Color | Scaling equation |
| --- | --- |
| Red | PWMRED = ((PWMmax X R data + 128) >> 8) X TC + 256) >> 9 (When SETUP1.TC_E = 1, another case see Table 5) |

Diagnostics

The ID804 provides built-in diagnostic functions for automatic detection of fault conditions.

If a fault is detected, the respective fault flag bit is set. The fault flag bit can only be cleared when the STATUS2 register has been read by the MCU (either directly or via status request bit), all flag bits have been cleared using CLRERROR, or the unit has received a RESET command. See Chapter 4.5 for more details.

If selected, the ID804 will automatically turn off the LED drivers when a failure is detected and change to SLEEP state to prevent damage to the part. This helps to ensure that communication is always possible through the entire daisy-chain.

- Overtemperature(OT) : This error occurs when the IC junction temperature rises above the overtemperature threshold. It can only be cleared once the temperature has fallen below the overtemperature release threshold. See Chapter 4.5 for details on controlling the threshold values.

- Undervoltage(UV) : This error occurs when the supply voltage(VDD) of the ID804 drops below the undervoltage threshold for a period longer than the deglitch time. Refer to the Chapter 3.3 for more details.

- LED open / short : This error occurs when the LED driver pin voltage remains above the LED-open threshold voltage or falls below the LED-short threshold voltage lasts for 2 consecutive EVENT_CYC times. These error conditions are tested individually for each channel, but only once after the GOACTIVE command has been received and the PWM value of the channel is set to a value higher than the minimum value. Refer to the Chapter 3.3 for more details.

- Communication Error : See Chapter 4.2.7.6.

- Communication Timeout : See Chapter 4.2.7.7

Fault Reaction

Table 7. Fault Reaction Table

| Fault | Detection | Fault bit | Configuration | Reaction |
| --- | --- | --- | --- | --- |
| VDD UV | VDD < Vuvth_lo | UV_FLT | UV_FLT_E=0 | Only Fault bit set |
|  |  |  | UV_FLT_E=1 | Fault bit set & LED drivers are disabled & Go to SLEEP state |
| OT | TJ > Temperature threshold | OT_FLT | OT_FLT_E=0 | Only Fault bit set |
|  |  |  | OT_FLT_E=1 | Fault bit set & LED drivers are disabled & Go to SLEEP state |
| LED OPEN | VLED < VTH_OPEN | OPEN_R/G/B | OS_FLT_E=0 | Only Fault bit set |
|  |  |  | OS_FLT_E=1 | Fault bit set & LED drivers are disabled & Go to SLEEP state |
| LED SHORT | VLED > VTH_SHRT | SHORT_R/G/B | OS_FLT_E=0 | Only Fault bit set |
|  |  |  | OS_FLT_E=1 | Fault bit set & LED drivers are disabled & Go to SLEEP state |
| COMMUNICATION ERROR | See Table 2 | COM_FLT | COM_FLT_E=0 | Only Fault bit set |
|  |  |  | COM_FLT_E=1 | Fault bit set & LED drivers are disabled & Go to SLEEP state |
| COMMUNICATION Timeout | See Table 3 | T_OUT | - | TBD |

ID804 Commands

The ID804 supports the following commands.

Table 8. ID804 Commands

| Command | Normal Command |  |  | SR (Status Request) Command |  |  | Data bits |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Encoding | Broadcast possible | Multicast possible | Encoding with SR | Broadcast possible | Multicast possible |  |
| RESET | 0xB1 | O | O |  | - | - | 0 |
| INITBIDIR | 0xB2 | X | X |  | - | - | 0 |
| CLRERROR | 0xB3 | O | O | 0xF3 | O | X | 0 |
| GOSLEEP | 0xB4 | O | O | 0xF4 | O | X | 0 |
| GOACTIVE | 0xB8 | O | O | 0xF8 | O | X | 0 |
| GODEEPSLEEP | 0xBC | O | O | 0xFC | O | X | 0 |
| SET_SETUP1 | 0x86 | O | O | 0xC6 | O | X | 12 |
| SET_SETUP2 | 0x87 | O | O | 0xC7 | O | X | 12 |
| SET_MCAST | 0x88 | X | X | 0xC8 | X | X | 24 |
| SET_RGB | 0xA0 | O | O | 0xE0 | O | X | 24 |
| SET_TEMPTH | 0x8A | O | O | 0xCA | O | X | 12 |
| SET_TEMPHYS | 0x8B | O | O | 0xCB | O | X | 12 |
| SET_CURR_MAX_LVL | 0x8F | O | O | 0xCF | O | X | 12 |
| SET_TEMP_LUT_TC1 | 0x90 | O | O | 0xD0 | O | X | 12 |
| SET_TEMP_LUT_TC2 | 0x91 | O | O | 0xD1 | O | X | 12 |
| SET_TEMP_LUT_TC3 | 0x92 | O | O | 0xD2 | O | X | 12 |
| SET_TEMP_LUT_TC4 | 0x93 | O | O | 0xD3 | O | X | 12 |
| SET_TEMP_LUT_TC5 | 0x94 | O | O | 0xD4 | O | X | 12 |
| SET_TEMP_LUT_TC6 | 0x95 | O | O | 0xD5 | O | X | 12 |
| SET_TEMP_LUT_TC7 | 0x96 | O | O | 0xD6 | O | X | 12 |
| SET_TEMP_LUT_TC8 | 0x97 | O | O | 0xD7 | O | X | 12 |
| SET_TEMP_LUT_TC9 | 0x98 | O | O | 0xD8 | O | X | 12 |
| SET_TEMP_LUT_TC10 | 0x99 | O | O | 0xD9 | O | X | 12 |
| READ_STATUS1 | 0x41 | O | X |  | - | - | 12 |
| READ_STATUS2 | 0x42 | O | X |  | - | - | 12 |
| READ_TEMP | 0x43 | O | X |  | - | - | 12 |
| READ_TEMPST | 0x44 | O | X |  | - | - | 24 |
| READ_VEXT_TM | 0x45 | O | X |  | - | - | 12 |
| READ_SETUP1 | 0x46 | O | X |  | - | - | 12 |
| READ_SETUP2 | 0x47 | O | X |  | - | - | 12 |
| READ_MCAST | 0x48 | O | X |  | - | - | 24 |
| READ_RGB | 0x60 | O | X |  | - | - | 24 |
| READ_TEMPTH | 0x4A | O | X |  | - | - | 12 |
| READ_TEMPHYS | 0x4B | O | X |  | - | - | 12 |
| READ_PWM_RED_VAL | 0x4C | O | X |  | - | - | 12 |
| READ_PWM_GREEN_VAL | 0x4D | O | X |  | - | - | 12 |
| READ_PWM_BLUE_VAL | 0x4E | O | X |  | - | - | 12 |
| READ_CURR_MAX_LVL | 0x4F | O | X |  | - | - | 12 |
| READ_TEMP_LUT_TC1 | 0x50 | O | X |  | - | - | 12 |
| READ_TEMP_LUT_TC2 | 0x51 | O | X |  | - | - | 12 |
| READ_TEMP_LUT_TC3 | 0x52 | O | X |  | - | - | 12 |
| READ_TEMP_LUT_TC4 | 0x53 | O | X |  | - | - | 12 |
| READ_TEMP_LUT_TC5 | 0x54 | O | X |  | - | - | 12 |
| READ_TEMP_LUT_TC6 | 0x55 | O | X |  | - | - | 12 |
| READ_TEMP_LUT_TC7 | 0x56 | O | X |  | - | - | 12 |
| READ_TEMP_LUT_TC8 | 0x57 | O | X |  | - | - | 12 |
| READ_TEMP_LUT_TC9 | 0x58 | O | X |  | - | - | 12 |
| READ_TEMP_LUT_TC10 | 0x59 | O | X |  | - | - | 12 |

If the ID804 receives a command that is not listed above, a communication error is raised.

RESET:

Performs a complete reset of one or all devices. The effect is identical to a power cycle (see Chapter 4.1.1).

All register values are set to their default values, all error flags are cleared, the communication mode detection is restarted, LED drivers are turned off, and the address is set to 0xFFF. The device enters the UNINIT state.

After issuing the RESET command, wait for at least 150μs (TBD) before sending out the next command.

INITBIDIR:

Initiates the automatic addressing of the chain.

The command shall be addressed to the first unit in the chain always with address 0x001. The last unit in the chain (indicated by the EOL mode, see Chapter 4.2.1.2) returns its address to the MCU. Subsequent commands should only be sent after the response has been received by the MCU.

Figure 15. Automatic addressing feature

If the maximum number of node addresses has been reached (0xFEE), the unit stops the initialization procedure and returns its address (0xFEE) to the MCU.

If a node is already initialized, the command is ignored but the telegram is forwarded with incremented address and the last unit returns its address. This can be used to selectively initialize individual units in the chain after a RESET or POR. In this case, do not initialize the chain with an address different from 0x001 as this could cause undefined behavior.

Note that, when the last unit is not in EOL mode (e.g., CAN-FD), it will not recognize itself as last unit. Then, the application needs to ensure that the chain length is correctly received by the MCU.

Uninitialized nodes interpret this command.

Note : This command is not allowed as broadcast. If the broadcast address or multicast address is used nevertheless, all units in the chain will be initialized to address 0x000 and no response telegram will be sent. Proper operation can be restored with a broadcast RESET followed by INITBIDIR with address 0x001.

Note : If the command is sent using the broadcast address 0x000 into an already addressed chain, all units will set the COM_FLT and the last unit will return the TEMP & STATUS2 register with address 0x000.

CLRERROR:

Clears all error flags in one or all devices. If an error condition still persists, the error flag is automatically set again.

If the CLRERROR with SR command received, the device returns the TEMP & STATUS2 register after the error flags have been cleared.

GOSLEEP :

Sends one or all devices into SLEEP state. See Chapter 4.1.1.

GOACTIVE :

Sends one or all devices into ACTIVE state. See Chapter 4.1.1.

GODEEPSLEEP :

Sends one or all devices into DEEPSLEEP state. Chapter 4.1.1.

SET_SETUP1 / READ_SETUP1 :

Write / Return the SETUP1 register

| Bit | [11] | [10] | [9] | [8] | [7] | [6] | [5] | [4] | [3] | [2] | [1] | [0] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | TC_E | OS_FLT_E | OT_FLT_E | UV_FLT_E | COM_FLT_E | CRC_E | CLK_INV | LG_E | F_PWM_DIV[3:0] |  |  |  |
| Default | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x0 |  |  |  |
| Access | RW | RW | RW | RW | RW | RW | RW | RW | RW |  |  |  |
| TC_E |  |  | 0: RED LED temperature compensation disable. 1: RED LED temperature compensation enable. |  |  |  |  |  |  |  |  |  |  |
| OS_FLT_E |  |  | 0: LED open/short fault flag, OS_FLT, is raised. 1: OS_FLT bit is raised, and LED drivers are disabled. |  |  |  |  |  |  |  |  |  |  |
| OT_FLT_E |  |  | 0: Overtemperature fault flag, OT_FLT, is raised. 1: OT_FLT bit is raised, and LED drivers are disabled. |  |  |  |  |  |  |  |  |  |  |
| UV_FLT_E |  |  | 0: Undervoltage fault flag, UV_FLT, is raised. 1: UV_FLT bit is raised, and LED drivers are disabled. |  |  |  |  |  |  |  |  |  |  |
| COM_FLT_E |  |  | 0: Communication fault flag, C_FLT, is raised. 1: C_FLT bit is raised, and LED drivers are disabled. |  |  |  |  |  |  |  |  |  |  |
| CRC_E |  |  | 0: CRC disabled. 1: CRC enabled. |  |  |  |  |  |  |  |  |  |  |
| CLK_INV |  |  | CLK polarity for MCU mode 1 : Inverted |  |  |  |  |  |  |  |  |  |  |
| LG_E |  |  | 0: Low Grayscale PWM disable. 1: Low Grayscale PWM enable. If RGB data ≤ 0x8, the PWM frequency is FPWM /4. |  |  |  |  |  |  |  |  |  |  |
| F_PWM_DIV[3:0] |  |  | PWM frequency divider. The PWM Frequency is activated in only active mode. FPWM = mclk / (F_PWM_DIV+1) |  |  |  |  |  |  |  |  |  |  |

SET_SETUP2 / READ_SETUP2 :

Write / Return the SETUP2 register

| Bit | [11] | [10] | [9] | [8] | [7] | [6] | [5] | [4] | [3] | [2] | [1] | [0] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  | VEXT_MON_E | EVENT_CYC | UV_LVL[1:0] |  | SH_LVL[1:0] |  |
| Default |  |  |  |  |  |  | 0 | 0 | 0x1 |  | 0x1 |  |
| Access |  |  |  |  |  |  | RW | RW | RW |  | RW |  |
| VEXT_MON_E |  |  | 0: EXT_TM pin voltage monitoring disable. 1: EXT_TM pin voltage monitoring enable. |  |  |  |  |  |  |  |  |  |  |
| EVENT_CYC |  |  | This bit determines a period to detect fault events or to update the ADC output value. If VEXT_MON_E '1', then the ADC update time will be doubled. 0: 15.625khz (typical 64us) 1: 31.25khz (typical 32us) |  |  |  |  |  |  |  |  |  |  |
| UV_LVL[1:0] |  |  | VDD UVLO detection level. 2'b00 : VUV_TH = 3.6V 2'b01 : VUV_TH = 3.8V 2'b10 : VUV_TH = 4.0V 2'b11 : VUV_TH = 4.2V (default) |  |  |  |  |  |  |  |  |  |  |
| SH_LVL[1:0] |  |  | LED Short detection level. 2'b00 : VS_TH = 3.75V. 2'b01 : VS_TH = 4.00V. (default) 2'b10 : VS_TH = 4.25V. 2'b11 : VS_TH = 4.50V. |  |  |  |  |  |  |  |  |  |  |

SET_MCAST / READ_MCAST :

Write / Return the MCAST register

| Bit | [23] | [22] | [21] | [20] | [19] | [18] | [17] | [16] | [15] | [14] | [13] | [12] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | MCAST_ADDR [15:12] |  |  |  |
| Default |  |  |  |  |  |  |  |  | 0x0 |  |  |  |
| Access |  |  |  |  |  |  |  |  | RW |  |  |  |
| Bit | [11] | [10] | [9] | [8] | [7] | [6] | [5] | [4] | [3] | [2] | [1] | [0] |  |
| Name | MCAST_ADDR [11:0] |  |  |  |  |  |  |  |  |  |  |  |  |
| Default | 0x00 |  |  |  |  |  |  |  |  |  |  |  |  |
| Access | RW |  |  |  |  |  |  |  |  |  |  |  |  |
| MCAST_ADDR [15:0] |  |  | Multicast group address. In case that the device address indicates the multicast group address. All devices received the command act like a broadcast and all device belonging to a multicast address should treat the command transaction. This register can be accessed by SET_MCAST and READ_MCAST command. [0] = 1 : Multicast group address 0xFEF [1] = 1 : Multicast group address 0xFF0 □□□ [15] = 1 : Multicast group address 0xFFE 12 bits device addr 0x000 → broad-cast addr 0x001~0xFEE → individual node addr 0xFEF~0xFFE → multi-cast addr 0xFFF → default addr |  |  |  |  |  |  |  |  |  |  |

SET_RGB / READ_ RGB :

Write / Return the RGB data register

| Bit | [23] | [22] | [21] | [20] | [19] | [18] | [17] | [16] | [15] | [14] |  | [13] |  | [12] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | R_DATA [7:0] |  |  |  |  |  |  |  | G_DATA [7:4] |  |  |  |  |  |  |
| Default | 0x00 |  |  |  |  |  |  |  | 0x00 |  |  |  |  |  |  |
| Access | RW |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |
| Bit | [11] | [10] | [9] | [8] | [7] | [6] | [5] | [4] | [3] |  | [2] |  | [1] |  | [0] |  |
| Name | G_DATA [3:0] |  |  |  | B_DATA [7:0] |  |  |  |  |  |  |  |  |  |  |  |
| Default | 0x00 |  |  |  | 0x00 |  |  |  |  |  |  |  |  |  |  |  |
| Access | RW |  |  |  | RW |  |  |  |  |  |  |  |  |  |  |  |
| R_DATA [7:0] |  |  | Red color data This data is converted into 12-bit PWM data toward a sink driver. R_PWM data = (R_DATA x PWMMAX*) + 128) >> 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G_DATA [7:0] |  |  | Green color data This data is converted into 12-bit PWM data toward a sink driver. G_PWM data = (G_DATA x PWMMAX*) + 128) >> 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B_DATA [7:0] |  |  | This data is converted into 12-bit PWM data toward a sink driver. B_PWM data = (B_DATA x PWMMAX*) + 128) >> 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |

*) PWMMAX : Data stored on OTP

SET_TEMPTH / READ_TEMPTH :

Write / Return the TEMPTH register

| Bit | [11] | [10] | [9] | [8] | [7] | [6] | [5] | [4] | [3] | [2] | [1] | [0] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  | TEMP_THRESHOLD[9:0] |  |  |  |  |  |  |  |  |  |
| Default |  |  | 0x000 |  |  |  |  |  |  |  |  |  |
| Access |  |  | RW |  |  |  |  |  |  |  |  |  |
| TEMP_THRESHOLD[9:0] |  |  | Overtemperature fault detection threshold. The OT_FLT is raised automatically the temperature increases above this value. |  |  |  |  |  |  |  |  |  |  |

SET_TEMPHYS / READ_TEMPHYS :

Write / Return the TEMPHYS register

| Bit | [11] | [10] | [9] | [8] | [7] | [6] | [5] | [4] | [3] | [2] | [1] | [0] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  | TEMP_HYSTERISIS[9:0] |  |  |  |  |  |  |  |  |  |
| Default |  |  | 0x000 |  |  |  |  |  |  |  |  |  |
| Access |  |  | RW |  |  |  |  |  |  |  |  |  |
| TEMP_HYSTERISIS[9:0] |  |  | Overtemperature fault release threshold. The OT_FLT can be cleared only when the temperature is below this value. |  |  |  |  |  |  |  |  |  |  |

SET_TEMP_LUT_TXx*) / READ_TEMP_LUT_TXx:

Write / Return the TEMP_LUT_TCx register

| Bit | [11] | [10] | [9] | [8] | [7] | [6] | [5] | [4] | [3] | [2] | [1] | [0] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  | TEMP_LUT_TCx[11:0] |  |  |  |  |  |  |  |  |  |
| Default |  |  | 0x0000 |  |  |  |  |  |  |  |  |  |
| Access |  |  | RW |  |  |  |  |  |  |  |  |  |
| TEMP_LUT_TCx[11:0] |  |  | 10bit TEMP LUT TCx data when the TEMPERATURE is in the range of (TC_BASE+TC_OFFSET*(x-1)+(x-1)) to TC_OFFSET, TEMP_LUT_TCx[9:0] is applied during TC equation |  |  |  |  |  |  |  |  |  |  |

*) : x = 1~10

SET_CURR_MAX_LVL / READ_CURR_MAX_LVL:

Write / Return the CURR_MAX_LVL register

| Bit | [11] | [10] | [9] | [8] | [7] | [6] | [5] | [4] | [3] | [2] | [1] | [0] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | R_CURR_MAX_LVL[3:0] |  |  |  | G_CURR_MAX_LVL[3:0] |  |  |  | B_CURR_MAX_LVL[3:0] |  |  |  |
| Default | 0x8 |  |  |  | 0x6 |  |  |  | 0x4 |  |  |  |
| Access | RW |  |  |  | RW |  |  |  | RW |  |  |  |
| R/G/B_CURR_MAX_LVL[3:0] |  |  | This determines the max current value of the output each (R/G/B) sink driver. 4'b0000: 1.44mA 4'b0001: 2.87mA 4'b0010: 4.31mA 4'b0011: 5.75mA 4'b0100: 7.18mA 4'b0101: 8.62mA 4'b0110: 10.05mA 4'b0111: 10.48mA 4'b1000: 12.92mA 4'b1001: 14.35mA 4'b1010: 15.78mA 4'b1011: 17.21mA 4'b1100: 18.65mA 4'b1101: 20.08mA 4'b1110: 21.51mA 4'b1111: 22.94mA |  |  |  |  |  |  |  |  |  |  |

READ_STATUS1 :

Return the STATUS1 register

| Bit | [11] | [10] | [9] | [8] | [7] | [6] | [5] | [4] | [3] | [2] | [1] | [0] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  | IC_STATE[2:0] |  |  | SIO2[1:0] |  | SIO1[1:0] |  |
| Default |  |  |  |  |  | 0x0 |  |  | 0x0 |  | 0x0 |  |
| Access |  |  |  |  |  | RO |  |  | RO |  | RO |  |
| IC_STATE[2:0] |  |  | Represent current IC state. 3'b000 : POR. (default) 3'b001 : Uninitialized. 3'b010 : Sleep. 3'b011 : Active. 3'b100 : Deepsleep. |  |  |  |  |  |  |  |  |  |  |
| SIO2[1:0] |  |  | Communication mode of SIO2. 2'b00 : LVDS. (default) 2'b01 : EOL. 2'b10 : MCU 2'b11 : CAN Value is set automatically at startup and after reset. |  |  |  |  |  |  |  |  |  |  |
| SIO1[1:0] |  |  | Communication mode of SIO1. 2'b00 : LVDS. (default) 2'b01 : EOL. 2'b10 : MCU 2'b11 : CAN Value is set automatically at startup and after reset. |  |  |  |  |  |  |  |  |  |  |

READ_STATUS2 :

Return the STATUS2 register

| Bit | [11] | [10] | [9] | [8] | [7] | [6] | [5] | [4] | [3] | [2] | [1] | [0] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | OTPCRC | T_OUT | OPEN_R | OPEN_G | OPEN_B | SHORT_R | SHORT_G | SHORT_B | OT_FLT | UV_FLT | CRC_FLT | COM_FLT |
| Default | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Access | RC | RC | RC | RC | RC | RC | RC | RC | RC | RC | RC | RC |
| OTPCRC |  |  | OTP error flag set during startup when a mismatch between the OTP values and the internal CRC is detected. Cleared after readout |  |  |  |  |  |  |  |  |  |  |
| T_OUT |  |  | Frame Time Out event flag is raised if the response is not arrived within a fixed time, and auto-cleared after readout. |  |  |  |  |  |  |  |  |  |  |
| OPEN_R |  |  | Red LED Open event flag is raised if VHD ≤ VOPEN for more than the deglitch time and the PWM data is not zero (not R/G/B DATA), and auto-cleared after readout. If the event flag is raised under OS_FLT_E ‘1’, then the LED sink driver is turned off. |  |  |  |  |  |  |  |  |  |  |
| OPEN_G |  |  | Green LED Open event flag is raised if VHD ≤ VOPEN for more than the deglitch time and the PWM data is not zero (not R/G/B DATA), and auto-cleared after readout. If the event flag is raised under OS_FLT_E ‘1’, then the LED sink driver is turned off. |  |  |  |  |  |  |  |  |  |  |
| OPEN_B |  |  | Blue LED Open event flag is raised if VHD ≤ VOPEN for more than the deglitch time and the PWM data is not zero (not R/G/B DATA), and auto-cleared after readout. If the event flag is raised under OS_FLT_E ‘1’, then the LED sink driver is turned off. |  |  |  |  |  |  |  |  |  |  |
| SHORT_R |  |  | Red LED Short event flag is raised if VHD ≥ VS_TH for more than the deglitch time and the PWM data is not zero (not R/G/B DATA), and auto-cleared after readout. If the event flag is raised under OS_FLT_E ‘1’, then the LED sink driver is turned off. |  |  |  |  |  |  |  |  |  |  |
| SHORT_G |  |  | Green LED Short event flag is raised if VHD ≥ VS_TH for more than the deglitch time and the PWM data is not zero (not R/G/B DATA), and auto-cleared after readout. If the event flag is raised under OS_FLT_E ‘1’, then the LED sink driver is turned off. |  |  |  |  |  |  |  |  |  |  |
| SHORT_B |  |  | Blue LED Short event flag is raised if VHD ≥ VS_TH for more than the deglitch time and the PWM data is not zero (not R/G/B DATA), and auto-cleared after readout. If the event flag is raised under OS_FLT_E ‘1’, then the LED sink driver is turned off. |  |  |  |  |  |  |  |  |  |  |
| OT_FLT |  |  | Over temperature fault flag is raised if TEMP value rises above OT_TH for more than the deglitch time, and auto-cleared after readout. If the fault flag is raised under OT_FLT_E ‘1’, then all LED sink drivers are turned off |  |  |  |  |  |  |  |  |  |  |
| UV_FLT |  |  | Undervoltage fault flag is raised if VDD drops below VUVLO for more than the deglitch time, and auto-cleared after readout. If the fault flag is raised under UV_FLT_E ‘1’, then all LED sink drivers are turned off. monclk = mclk / 2^8 = 15.5us (typ) monclk deactivated in deep sleep mode (working in active and sleep) deglitch time : the event flag is raised in case the event condition lasts for monclk * 4 monclk * 4 = 62us (typ) |  |  |  |  |  |  |  |  |  |  |
| CRC_FLT |  |  | CRC fault flag is raised if CRC is wrong under CRC_E ‘1’, and auto-cleared after readout. |  |  |  |  |  |  |  |  |  |  |
| COM_FLT |  |  | Communication fault flag is raised if any communication error is occurred, and auto-cleared after readout. If the fault flag is raised under COM_FLT_E ‘1’, then all LED sink drivers are turned off. |  |  |  |  |  |  |  |  |  |  |

READ_TEMP :

Return the TEMP register

| Bit | [11] | [10] | [9] | [8] | [7] | [6] | [5] | [4] | [3] | [2] | [1] | [0] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  | TEMPERATURE[9:0] |  |  |  |  |  |  |  |  |  |
| Default |  |  | 0x000 |  |  |  |  |  |  |  |  |  |
| Access |  |  | RO |  |  |  |  |  |  |  |  |  |
| TEMPERATURE[9:0] |  |  | Temperature data |  |  |  |  |  |  |  |  |  |  |

READ_TEMPST :

Return the TEMP & STATUS2 register

READ_VEXT_TM :

Return the VEXT_TM register

| Bit | [11] | [10] | [9] | [8] | [7] | [6] | [5] | [4] | [3] | [2] | [1] | [0] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  | VEXT_TM[9:0] |  |  |  |  |  |  |  |  |  |
| Default |  |  | 0x000 |  |  |  |  |  |  |  |  |  |
| Access |  |  | RO |  |  |  |  |  |  |  |  |  |
| VEXT_TM[9:0] |  |  | EXT_TM pin voltage. |  |  |  |  |  |  |  |  |  |  |

READ_PWM_RED_VAL :

Return the PWM_RED_VAL register

| Bit | [11] | [10] | [9] | [8] | [7] | [6] | [5] | [4] | [3] | [2] | [1] | [0] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | PWM_RED_VAL[11:0] |  |  |  |  |  |  |  |  |  |  |  |
| Default | 0x0000 |  |  |  |  |  |  |  |  |  |  |  |
| Access | RO |  |  |  |  |  |  |  |  |  |  |  |
| PWM_RED_VAL[11:0] |  |  | 12bit RED PWM calculated data value |  |  |  |  |  |  |  |  |  |  |

READ_PWM_GREEN_VAL :

Return the PWM_GREEN_VAL register

| Bit | [11] | [10] | [9] | [8] | [7] | [6] | [5] | [4] | [3] | [2] | [1] | [0] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | PWM_GREEN_VAL[11:0] |  |  |  |  |  |  |  |  |  |  |  |
| Default | 0x0000 |  |  |  |  |  |  |  |  |  |  |  |
| Access | RO |  |  |  |  |  |  |  |  |  |  |  |
| PWM_GREEN_VAL[11:0] |  |  | 12bit GREEN PWM calculated data value |  |  |  |  |  |  |  |  |  |  |

READ_PWM_BLUE_VAL :

Return the PWM_BLUE_VAL register

| Bit | [11] | [10] | [9] | [8] | [7] | [6] | [5] | [4] | [3] | [2] | [1] | [0] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | PWM_BLUE_VAL[11:0] |  |  |  |  |  |  |  |  |  |  |  |
| Default | 0x0000 |  |  |  |  |  |  |  |  |  |  |  |
| Access | RO |  |  |  |  |  |  |  |  |  |  |  |
| PWM_BLUE_VAL[11:0] |  |  | 12bit BLUE PWM calculated data value |  |  |  |  |  |  |  |  |  |  |

Typical Application Circuit

Figure 22.

Layout Guidelines

Package Information

Figure 16. Package of Dimension